Readers’ suggested software for the LED flasher
Ingenuity Unlimited, December 2012 issue

;Program to modulate the blue Christmas lights on tree outside.

;Sept 2012

;KAN

List P=12F629

#include <P12F629.inc>

__CONFIG  _CP_OFF & _BODEN_OFF & _PWRTE_OFF & _INTRC_OSC_NOCLKOUT & _MCLRE_OFF & _CPD_OFF&_WDT_OFF

#define	BANK0	bcf	STATUS,RP0

#define	BANK1	bsf	STATUS,RP0

#define	Lon	bsf	GPIO,5

#define	Loff	bcf	GPIO,5

dot	MACRO			;'dot' to save a bit of writing

call	Dot

ENDM

dash	MACRO

call	Dash

ENDM

sp	MACRO

call	Sp

ENDM

;RAM starts at 20H

temp1	EQU	20H

temp2	equ	21H

ont	equ	22H

offt	equ	23H

lp1	equ	24H

lp2	equ	25H

temp	equ	26H

;====================code======================

ORG	0

goto	Init	;initialisation

ORG	4	;interrupt (not used)

;---------------------------------------------------

; Initialise the system

;Use the 4MHz internal clock

;set the port directions,

Init	BANK1

call	3FFH	;get osc cal value

movwf	OSCCAL

movlw	b'001000'	;GP3 to intput

movwf	TRISIO	;set directions

clrf	WPU	;weak pull ups off

clrf	IOC	;interrupt on change-not used

movlw	07H

movwf	OPTION_REG	;Timer 0 256:1

BANK0

movlw	07H

movwf	CMCON	;comparator off, digital I/O

;Ready to begin

Bgn	Lon

btfss	GPIO,3	;Test S1. LEDs on all time if closed

goto	Bgn

movlw	20H	;Now run the slow dimming  a few times

movwf	temp

st	call	 dim1

decfsz	temp,f

goto	st

call	waitT1	;

call	waitT1	;On 1 sec at start

dot	;H  Now spell out the Morse Code value of each letter

dot

dot

dot

sp	;With a pause between letters

dot	;A

dash

sp

dot	;P

dash

dash

dash

sp	;etc. (Rest of message omitted)

;end of Christmas message, now a long 'on' period

Lon

movlw	40H

movwf	temp

nx	call	waitT1

decfsz	temp,f

goto	nx

goto	Bgn

;SUBROUTINESS

;-------------------------------------------------------------

Dot	Lon

call	waitTs	;Short ON

Loff

call	waitTs	;pause

return

Dash	Lon

call	waitT1	;Long ON

Loff

call	waitTs	;pause

return

Sp	call	waitT1	;inter-character delay

return

;.................................

;To give a long delay (535ms)

LoadT1	movlw	01H

movwf	TMR1H		;load count high

bcf	PIR1,TMR1IF	;clear the flag

movlw	35H	;set to 8:1 prescale clock i.e. 8us

movwf	T1CON	;load and start T1

return

;

waitT1	call	LoadT1

wt1	btfsc	PIR1,TMR1IF

return

goto	wt1

LoadTs	movlw	81H

movwf	TMR1H		;load count high

bcf	PIR1,TMR1IF	;clear the flag

movlw	35H	;set to 8:1 prescale clock i.e. 8us

movwf	T1CON	;load and start T1

return

waitTs	call	LoadTs

wts	btfsc	PIR1,TMR1IF

return

goto	wts

;..................................................................

;dim slowly from bright to dim and then back again

dim1	clrf	ont

clrf	offt

movlw	02H	;2 repeats at each setting

movwf	lp1	;inner loop

lpi	movf	lp1,w

movwf	temp2	;load temp2

decf	ont,f	;1st value is FF

incf	offt,f	;1st value is 01

movlw	0E0H	;limit

subwf	offt,w

btfss	STATUS,Z		; test for equality

goto	p2

goto	lpi2

p2	Lon

call	x1delay 	;on time

Loff

call	x2delay 	;off time

decfsz	temp2,f

goto	p2	;loop with this setting temp2  times (lp1)

goto	lpi

lpi2	movf	lp1,w

movwf	temp2	;load temp2

incf	ont,f

decf	offt,f

movlw	01H	;limit

subwf	offt,w

btfss	STATUS,Z		; test for equality

goto	p22

goto	px	;done

p22	Lon

call	x1delay 	;on time

Loff

call	x2delay 	;off time

decfsz	temp2,f

goto	p22	;loop with this setting temp2  times (lp1)

goto	lpi2

px	return

;variable on and off timedelays for dimming and brightening

x1delay	movf 	ont,w	;ont is variable parameter

movwf	temp1

dd	decfsz	temp1,f

goto	dd

return

;------------------------------------

x2delay	call	fd	;fixed delay

movf	offt,w	;offt is a variable

movwf	temp1

dd2	decf	temp1,f

btfsc	STATUS,Z

return

nop

nop

goto	dd2

retlw	0

fd	movlw	20H	;fixed off dely

movwf	temp1

ddd	decfsz	temp1,f

goto	ddd

return

END